# ZKX6040A

Single Energy X-ray Inspection System

ZKX6040A is a single-energy X-ray screening system that provides a highly economical visual inspection solution for carry-on luggage.

clear scanning image, which allows operators to identify potential threat items visually.

# Features

Pseudo-color scanning image Bi-directional

Foldable Keyboard

Input/Output Roller Conveyor Energy Saving Mode

# Options

Operator Desk Hard Disk

Video Surveillance

Display of Cluttered Bag Images together with the Status of the Walk-though Metal Detector

# Software Features

Perfect View (PV) Black / White (B/W) Pseudo Colors (PS) Inverse View (IN) High Penetration (HP)

Real-time Image Processing and Image Zoom (64x) Image of Previous Baggage

Programmable Function Keys Baggage Counter

Date / Time Indicator

Threat Image Projection (TIP)

# Speci cations

## Technical Characteristics

Tunnel Dimensions (W*H)  600*400 (mm)

Conveyor Speed 0.3m/sec

Load on Conveyer  180kg (evenly distributed)

Conveyor Height 680 mm

HorizontalΦ1.0mm VerticalΦ2.0mm

Steel Penetration 8mm Wire Resolution 34AWG Single-energy	Yes

### Number of Colors in X-ray Image  3

Dimensions (L*W*H) 1701*910*1305 (mm)	 Dimensions with Packing(L*W*H) 2070*1150*1400 (mm)	 Net Weight 380 kg

Weight with Packing  480 kg

Film Safety

ASA/ISO1600 standard

### Single Inspection Dosage Rate  ≤1.0µGy

Supported Software ZKBio SIS

Communication Interface RJ45(TCP/IP)

Radiation Leak Dosage	0.25µ Gy/h (5cm from the surface)

# Dimensions (mm)

## Operating Environment

Operating Temperature -10°C to +45°C

Storage Temperature -20°C to +60°C

Humidity 5-95%, non-condensing

Power Supply

230 VAC +/- 10% / 60/50 Hz (EU)

110 VAC +/- 10% / 60/50 Hz (US)

Power Consumption Max. 0.3kVA

Sound Pressure 54dB

## Computer Speci cations

Processor Inter core i3-4100 Monitor 21.5” LED display Memory Onboard 4 GB RAM

Rom	128G

USB Port 4 ports

Max. Storage Up to 10,000 scanned images

## X-ray Generator

Tube Current 0.6 mA

Anode Voltage 80kV rated

Generator Cool Seal oil cooling / 100%

Orientation Vertically upwards

910

600

1701

400

2736

1305

V6.1 10/24/2023

ZKTECO CO.,LTD.	www.zkteco.com	E-mail: sales@zkteco.com	 Copyright © 2023 ZKTECO CO., LTD. All rights reserved.