# BLADE100100

X-ray Baggage Inspection System

BLADE100100 is an X-ray baggage inspection system with a tunnel size of 1010*1010(mm) which provides an effective inspection on big baggage and other items. It helps to identify suspicious items such as weapons, liquids, knives, fire guns, bombs, toxic substances, substances, ammunition and other dangerous objects which may lead to potential safety hazard by atomic number identification.

High image quality in combination with automatic identification of suspicious objects allows the operator to quickly and effectively evaluate any luggage content.

# Features

- Real-time detection to aid operators in identifying suspicious item

- Typical steel penetration up to 35mm

- Uninterruptible Power Supply (UPS)

- Equipped with foldable rack

- Adjustable conveyor Speed

- Real-time Remote Monitoring System

- Luggage image and Walk Through Metal Detector status display on the same screen

- Threat Image Projection (TIP)

- Operator desk with two monitors

# Options

- Video Surveillance	• Gray Paint

( GL Exclusive Feature)

295

Technical Characteristics:

Tunnel Dimensions(W*H): 1010mm*1010mm Conveyor Speed: 0.20 m/sec ~ 0.40m/sec Load on Conveyer:

Max 200 kg (evenly distributed) Space Definition: HorizontalΦ1.0mm VerticalΦ2.0mm

Steel Penetration: 35mm (guaranteed);

38mm (typical) Wire Resolution: 38AWG (guaranteed);

40AWG (typical) Dual-energy Detector: Yes Number of Colors in X-ray Image: 3 Dimensions(L*W*H): 3200mm*1482mm*1917mm

Dimensions of Operator Desk (L*W*H): 797mm*789mm*1140mm

Package Dimensions (L*W*H): 2900mm*1500mm*2100mm

Weight: 1260kg + 70kg (Operator Desk) Package Weight: 1530kg

Film Safety: ASA/ISO1600 standard of film safety Single Inspection Dosage Rate: ≤1.0µGy

# Dimensions (mm)

3200

2301

1482

1010

1917

324

324

266

1010

4282

## Operating Environment:

Operating Temperature: -10°C to +45°C Storage Temperature: -20°C to +60°C Humidity: 5-95%, non condensing

Power Supply:230 VAC +/- 10% / 60/50 Hz (EU) 110 VAC +/- 10% / 60/50 Hz (US)

Power Consumption: Approx.1.0kVA Sound Pressure: <55.8dB (A)

## X-ray Generator:

Tube Current: 0.5-1.2 mA (adjustable) Anode Voltage: 150-170 kV (adjustable) Generator Cool: Seal oil cooling / 100%

## Computer Speci cation:

Processor: I7-7700 Monitor: 21” LED color Memory: 8 GB DDR4

Hard Drive: 240GB SSD+1TB HDD

Graphic Card: NVIDIA GeForce GTX 1660 Ti USB Port: 4 ports

## Software Features:

Perfect View (PV) Black / White (B/W) Pseudo Colors (PS) Inverse View (IN) High Penetration (HP)

Real-time Image Processing Image Zoom(64x)

Image of Previous Baggage Programmable Function Keys Baggage Counter

Date / Time Indicator

Threat Image Projection (TIP) AI Intelligent Analysis

Platform software：ZKBio SIS

Status Monitoring of Walk Through Metal Detector

ZKTECO CO.,LTD.

www.zkteco.com

E-mail: sales@zkteco.com

Copyright © 2023 ZKTECO CO., LTD. All rights reserved. V2.0 2023.10.23