Dual Energy X-ray Inspection System ZKX5030C

Highlights

high Performance X-ray detector True-color scanning image Widely usable for small parcels

Excellent, ergonomic industrial design

Standard

Energy Saving Mode

Optional

Bidirection scan Fingerprint Console Board Console Desk

Introduction

ZKX5030C X-ray inspection system increases the operator’s ability to identify potential threats; the device is designed to scan briefcases, carry-on baggage, small cargo parcels.

ZKX5030C uses reliable high quality X-ray generator. With the superb image algorithm, ZKX5030C could o er clear scanning image, which allows operators to identify potential threat items visually.

ZKX5030C has innovative biometric identify function for operators, improving the security of system and preventing operator from forgetting password.

With ergonomic modern design, ZKX5030C could help operators to identify suspicious items fast and accurately.

Sp	cations

| Tunnel Size | W507mm × H305mm |
| --- | --- |
| Speed | 0.20 m/s |
| Height of Transmission Belt | 730 mm |
| Maximum Load | ≤120 kg (Adequate distribution) |
| Wire Resolution | 36 AWG |
| Space De nition | Horizontal Φ1.0mm \ Vertical Φ2.0mm |
| Penetrate De nition | 32 AWG |
| Penetration | 32mm Steel board |
| Monitor | 21.5 inch LED |
| System Function | High density alarm, Explosive/Drug auxiliary detect, TIP, Luggage counter, System running timer, X-ray emitting timer, Training, 64 times continuous zoom in |
| Film Safety | ASA/ISO1600 standard of lm safety |

X-ray System

| Tube Voltage | 140 KV |
| --- | --- |
| Cooling | Seal oil cooling / 100% |
| Single Inspection Dosage Rate | ≤1.0 µ Gy |
| Radiation Leak Dosage | 0.1µ Gy/h (5cm from the surface) |

Installation Sp	cation

| Size | L1701mm x W796mm x H1150mm |
| --- | --- |
| Package Size | L1900mm × W990mm × H1485mm |
| Package Weight | 450 KG |
| Power Consumption | 1 kVA |
| Noise | 53.8 dB(A) |

Dimension

796

500

1701

1535

300

2391

1150

650

V1.4 2024.03.25

ZKTECO CO.,LTD.	www.zkteco.com	E-mail: sales@zkteco.com	Copyright © 2024 ZKTECO CO., LTD. All rights reserved.