# BLADE6040

Screener Assist

X-ray Baggage Inspection Systems

BLADE6040 is a X-ray baggage inspection which has a tunnel size of 610 mm by 420 mm and can provide the e ective inspection of mail, hand-held baggage, luggage and other items. It allo	weapons,liquids, explosives, drugs, knives	re guns, bombs, toxic substances		s, ammunition, and dangerous objects, which are safety hazard b			ective atomic number.

High image quality in combination with aut	cts allows the operator to quickly and e ectively evaluate any luggage content.

# Features	Options

- Real-time Detection to Aid Operators in Identifying Suspicious

- High Performance – Typical Penetration up to 40 mm of Steel

- Startup with Fingerprint

- Four Layers of Lead Curtain

- Uninterruptible Power Supply

- Equipped with Input & Output Rollers

- Adjustable Conveyor Speed

- Real Time Remote Monitoring System

- Luggage Image and Walk Through Metal Detector Status Display on the Same Screen

- Threat Image Projection (TIP)

- Operator Desk

- Video Surveillance

- Gray Paint

- Guard Plate of Rollers (Two layers of lead curtain)

- Eyes on Function (Eye tracking technology to fol low the users eye as they operating x-ray inspec tion.There will be a prompt when the operator’s eyes leave the screen.)

( GL Exclusive Feature)

## Technical Characteristics:

Tunnel Dimensions(W*H): 610mm*420 mm Conveyor Speed: 0.20 m/sec ~ 0.40m/sec Load on Conveyer:

Max 200 kg (Evenly Distributed)

HorizontalΦ1.0mm VerticalΦ1.0mm Steel Penetration: 38mm Guaranteed; 40mm Typical

Wire Resolution: 38AWG Guaranteed; 40AWG Typical

Dual-energy detector: Yes

Number of Colors in X-ray Image: 3 Dimensions(L*W*H): 1987.5*1023.9*1317mm

Package Dimensions(L*W*H): 2100mm*1200mm*1600 mm Package Weight: 750kg

Film Safety: ASA/ISO1600 standard of lm safety Single Inspection Dosage Rate: ≤1.0µGy Radiation Leak Dosage 0.1µ Gy/h (5cm from the surface)

# Dimensions (mm)

## Operating Environment:

Operating Temperature: -10°C to +45°C Storage Temperature: -20°C to +60°C Humidity: 5-95%, non condensing

Power Supply:230 VAC +/- 10% / 60/50 Hz (EU) 110 VAC +/- 10% / 60/50 Hz (US)

Power Consumption: 700W Sound Pressure: <51.8dB (A)

## X-ray Generator:

Tube Current: 0.5-1.0 mA(adjustable) Anode Voltage: 150-170 kV(adjustable) Generator Cool: Seal oil cooling / 100%

## Computation:

Processor: I7-7700 Monitor: 21” LED color Memory: 8 GB DDR4

Hard Drive: 240GB SSD+2TB HDD

Graphic Card: NVIDIA GeForce GTX 1660 Ti USB Port: 4 ports

## Software features:

Perfect View (PV) Black / White (B/W) Pseudo Colors (PS) Inverse View (IN) High Penetration (HP)

Real Time Image Processing Image Zoom(128x)

Image of Previous Baggage Programmable Function Keys Baggage Counter

Date / Time Indicator

Threat Image Projection (TIP)

Automated threat identi cation software Platform software：ZKBio SIS

Monitor the status of walk through metal detector

ZKTECO CO.,LTD.

www.zkteco.com

E-mail: sales@zkteco.com

Copyright © 2023 ZKTECO CO., LTD. All rights reserved. V2.2 2023.12.11